SPI protocol:

Master section:

module spi_master(input clk,

input rst,

input start,

input [7:0]data, //mosi

input miso,  //data sent by slave --->master

output reg mosi,  //master ---->slave

output reg sclk,

output reg ss,

output reg [7:0]data_out,

output reg done

);

reg [2:0]bit_cnt;

reg [7:0]shift_reg;  //load and sent to slave

reg [7:0]recv_reg;  //store the data sent from slave

parameter IDLE=0,

LOAD=1,

TRANSFER=2,

DONE=3; //11

reg [1:0]state;

always @(posedge clk or posedge rst)begin

if(rst)begin

sclk<=0;

mosi<=0;

ss<=1;

done<=0;

state<=IDLE;

bit_cnt<=0;

shift_reg<=0;

recv_reg<=0;

data_out<=0;

end

else begin

case(state)

IDLE:begin

ss<=1;

done<=0;

if(start)begin

state<=LOAD;

end

else

state<=IDLE;

end

LOAD:begin

ss<=0;  //enable the slave

sclk<=0;

shift_reg<=0;

bit_cnt<=3'd7;  //MSB

state<=TRANSFER;

end

TRANSFER:begin

sclk<=~clk;

if(sclk==0)begin  //neg edge

mosi <= shift_reg[bit_cnt];

end

else begin

recv_reg[bit_cnt]<=miso;  //receive data

if(bit_cnt==0)begin

state<=DONE;

end

else begin

bit_cnt <=bit_cnt-1;

state<=TRANSFER;

end

end

end

DONE:begin

ss<=1;

done<=1;

data_out<=recv_reg;

state<=IDLE;

end

endcase

end

end

endmodule

Slave-Section

module spi_slave(input sclk,rst,

input ss,

input mosi,//master data

output reg miso,

output reg [7:0]slave_recived  );//master data

reg [7:0] shift_in;//to store data from master

reg [7:0] shift_out=8'b11011001; //to send to the master

reg [2:0] bit_cnt;

always @(negedge ss or posedge rst)begin  //start transaction

if(rst)begin

bit_cnt <= 3'd7;

miso <= 0;

end

else begin

bit_cnt <= 3'd7;

miso <= shift_out[7];

end

end

always @(posedge sclk or posedge rst)begin  //sampling (read)

if (rst)begin

shift_in <= 0;

slave_recived <=0;

bit_cnt <= 7;

end

else if(!ss)begin

shift_in[bit_cnt] <= mosi;  //receive data from master

if(bit_cnt ==0)begin

slave_recived <= {shift_in[7:1],mosi};

end

else if(bit_cnt >0)

bit_cnt <= bit_cnt-1;

end

end

always @(negedge sclk or posedge rst)begin

if(rst)begin

miso <=0;

end

else if(!ss)begin

miso <= shift_out[bit_cnt];   //sending data to master

end

else

miso<=0;

end

endmodule

TB:

module top_tb();

reg clk,rst,start;

reg [7:0]master_data_in;

wire [7:0]master_recived;

wire [7:0]slave_recived;

wire mosi,miso,sclk,ss,done;

spi_master master(.clk(clk),

.rst(rst),

.start(start),

.data(master_data_in),

.miso(miso),

.mosi(mosi),

.sclk(sclk),

.ss(ss),

.data_out(master_recived),

.done(done));

spi_slave slave(.sclk(sclk),

.rst(rst),

.ss(ss),

.mosi(mosi),

.miso(miso),

.slave_recived(slave_recived));

always #5 clk=~clk;

initial begin

clk=0;rst=1;start=0;

master_data_in = 8'b00000000;

$monitor("master received data:%b | slave received:%b",

master_recived,slave_recived);

#20 rst=0;

master_data_in=8'b11111111;

start=1;

#10 ; start=0;

wait(done);

#10;

master_data_in=8'b01101101;

start=1;

#10;start=0;

wait(done);

#50 $finish;

end

endmodule